Integration Note

| Manufacturer: | SmartRF |
| --- | --- |
| Model Number(s): | SmartRF Pro |
| g! Core Module Version: | 8.7.602 |
| Driver Developer: | Zoom Digital Home |
| Document Revision Date: | 09 September 2022 |

Overview

This driver is to integrate SmartRF Pro gateway to control blind/shades with RF 433 Mhz or Somfy RTS motors.

Supported Features

- Open/Close/Stop: Control the open, close and stop of blind/shades motors.

Any feature not specifically noted as supported should be assumed to be unsupported.

Driver License

This driver works for a trial period. After that it’s requires a license key to function.

Pricing and license keys can be obtained contacting us by drivers@zoomdigitalhome.com.br. License keys are issued on a per controller basis. You only need a single license key for multiple instances on the same controller.

Dealer Support

Any help or support questions, please, contact us by drivers@zoomdigitalhome.com.br.

Device Setup

Set up your device as recommended by manufacturer’s instructions.

Pay attention to IP address and communication port.

Set a static IP or reserve it on DHCP server.

Installation Process

- Please check the latest driver version on www.zoomdigitalhome.com.br/drivers

- Under the Lighting tab in configurator, add a new Lighting Interface.

- Use the Search Folder button to point Configurator to the location of you saved the driver.

- Select the driver and click OK.

- Set up SmartRF Pro settings:

  - Serial Number: inform the serial number of device, see the product tag.

  - PWD: inform the password, on product tag too.

  - IP Address: set the IP of device.

  - Port: set the TCP port of device (default 80).

- Under Lightning Devices, add New Device.

- Set a Device Name and select Device Type as Shade.

- On new device created, set up:

  - Chanel – motor channel

  - Open ID – id of button to open the blind/shade (default 1)

  - Stop ID – id of button to open the blind/shade (default 2)

  - Close ID – id of button to open the blind/shade (default 3)

Common Mistakes

- Incorrect IP address or TCP port settings.

- Incorrect Channel ID or Button ID settings.

ELAN Developer Partner Information

- This ELAN driver was written and supported by:

- www.zoomdigitalhome.com.br

- drivers@zoomdigitalhome.com.br